PaceAI

AI-Powered Real-Time Running Coach

System Specification v9.3

Girish · Mumbai · April 2026

# 1. What Is PaceAI

PaceAI is a DIY AI-powered real-time running coach. It combines a custom hardware foot pod strapped to the ankle, a Garmin FR245 heart rate monitor, phone GPS, and Claude AI to deliver live audio coaching through earphones during runs.

The system is built in three integrated layers:

- Hardware: ESP32 + MPU6050 foot pod + TP4056 + LiPo 500mAh battery

- Mobile App: Progressive Web App (PWA) hosted on GitHub Pages (girishk75.github.io/Pace)

- AI Coach: Claude Sonnet API delivering real-time spoken coaching

# 2. Hardware Stack

## 2.1 ESP32 Foot Pod

| Component | Detail |
| --- | --- |
| Microcontroller | ESP32 DevKit |
| Sensor | MPU6050 accelerometer/gyroscope |
| Power | LiPo 500mAh + TP4056 charger + slide switch |
| BLE Device Name | PaceAI-FootPod |
| Service UUID | 4fafc201-1fb5-459e-8fcc-c5c9c331914b |
| Characteristic UUID | beb5483e-36e1-4688-b7f5-ea07361b26a8 |
| Data Format | "cadence,impact,gct,steps" (CSV string) |
| Broadcast Rate | Every 1 second |
| Sampling Rate | 100Hz (every 10ms) |
| Firmware Version | v1.1 |
| Mounting Position | Ankle (under sock) |

## 2.2 Garmin FR245

- Broadcasts HR using standard BLE heart_rate service

- heart_rate_measurement characteristic

- Virtual Run mode required for BLE broadcast

- Confirmed working — avg match within 1-3bpm

## 2.3 Phone GPS

- Uses watchPosition() + getCurrentPosition() pre-warm

- Sliding 5-position haversine window for pace smoothing

- coords.speed used for first 1-2 fixes only

- Accuracy threshold: 150m (relaxed for Mumbai urban conditions)

- Minimum movement between fixes: 3m

- Pace bounds filter: 3:30–15:00/km (210–900 secs/km)

- Staleness detection: gpsPace flagged stale after 15 seconds without update

## 2.4 Power System

- LiPo 500mAh at 3.7V → approximately 3 hours runtime

- TP4056 module for safe LiPo charging via USB-C

- Slide switch between TP4056 OUT+ and ESP32 VIN

- Red LED = charging, Blue LED = fully charged

- Charging time: approximately 35-40 minutes from empty

# 3. App Stack

| Property | Value |
| --- | --- |
| Type | Single HTML file Progressive Web App (PWA) |
| Hosting | GitHub Pages — girishk75.github.io/Pace |
| Version | v9.3 |
| Framework | Vanilla JavaScript + CSS (no frameworks) |
| BLE | Web Bluetooth API |
| GPS | Geolocation API |
| Voice | Web Speech API |
| Storage | localStorage (runs, coach log, API key) |
| AI | Claude Sonnet API (claude-sonnet-4-20250514) |
| Screen Lock | Wake Lock API |

# 4. App Features

## 4.1 Screens

- Setup — configure run, connect BLE devices, view history

- Live Run — real-time metrics display during run

- Run Shield — fullscreen tap-lock overlay during run

- Paused — pause state with resume/end options

- Done — post-run summary with export buttons

- History — all runs with CSV export

- Settings — API key management

## 4.2 Live Metrics Displayed

- Elapsed time (large clock)

- Distance (km)

- Pace (min/km)

- Heart rate + HR zone (Z1–Z5) with animated zone strip

- Cadence (steps per minute)

- Steps (total, offset-corrected per run)

- Impact G (force per footstrike)

- Ground Contact Time (milliseconds)

- Composite fatigue index (0–10 scale)

- Fatigue breakdown: HR drift, cadence, GCT, impact components

- Live predictions: estimated finish, remaining distance, avg pace, HR%, fatigue risk

## 4.3 Simulated Data Indicators

Every metric clearly shows its data source. When hardware is not connected, the app falls back to simulation and shows:

- ⚡ SIM PACE / ⚡ sim km — when GPS not locked

- ⚡ Sim HR — when Garmin BLE not connected

- ⚡ Simulated — on cadence, steps, impact, GCT cards when foot pod not connected

- Source label in predictions panel updates dynamically (e.g. 'BLE HR · GPS ±8m')

## 4.4 Run Shield

Fullscreen overlay to prevent accidental button presses when phone is in waistband or pocket.

- Activates automatically 3 seconds after run start

- Shows: clock, distance, pace, HR — nothing else

- Hold anywhere for 2 seconds to unlock (green progress bar fills)

- Lock icon (🔒) in live header to re-engage anytime

- Disabled on pause/finish automatically

- Uses position:fixed, z-index:999 — covers all buttons including Pause/End

## 4.5 Wake Lock

- Acquired on run start — prevents screen auto-dimming

- Released on pause and finish

- Re-acquired on resume and visibilitychange event

- Cannot prevent manual screen lock (Android limitation — hardware button overrides)

- OnePlus 11R max screen timeout is 30 minutes — set this before runs

# 5. AI Coaching

## 5.1 Trigger Events

| Trigger | Condition |
| --- | --- |
| Run start | 3 seconds after tapping Start |
| 2-min check-in | Every 120 seconds elapsed |
| km milestone | Each km completed (1km, 2km, etc.) |
| HR Zone 4 entry | HR crosses into Z4 for first time |
| HR Zone 5 | Every 30 seconds in Z5 |
| Pace too slow | More than 20s/km behind target, every 75s |
| Pace too fast | More than 20s/km ahead of target, every 75s |
| Low cadence | Below 165spm, every 60s |
| High impact | Above 2.8G, every 90s |
| High fatigue | Fatigue index above 7/10, every 60s |

## 5.2 Prompt Context

Each AI call includes:

- Trigger reason (e.g. '5km milestone', '2-min check-in')

- Distance covered and remaining

- Elapsed time

- Current pace vs target pace (difference in seconds)

- HR, HR zone, percentage of max HR

- Cadence, GCT, impact force

- Composite fatigue + all four components

- Run type (easy/tempo/long/race/intervals)

- Weather condition

- Full runner profile (323 runs, 3043km, marathon finisher, Mumbai, max HR 185)

## 5.3 Voice Delivery

- Spoken via Web Speech API — delivered to connected earphones

- Rate 0.92, Pitch 1.0, Volume 1.0

- Audio indicator bar shown while speaking

- 🔊 Mute button in live header — toggles voice on/off

- Cancels current speech on mute or new message

# 6. Coach Log (Cross-Run)

Every AI coaching response is logged to a persistent cross-run database in localStorage (key: paceai_coach_log_v1). This accumulates across all runs permanently.

## 6.1 Fields Per Event (23 total)

| Category | Field | Description |
| --- | --- | --- |
| Identity | runId, runDate, runTime | Unique run ID + timestamps |
| Config | runType, weather, targetDist, targetPace | Run configuration |
| Trigger | trigger, adviceType | What caused the AI to fire |
| State | elapsedSecs, elapsedDisplay, distKm | Position in run |
| Pace | paceDisplay, paceDiffSecs | Actual pace vs target |
| Biometrics | hr, hrZone, cadence, gct, impact | All sensor values |
| Fatigue | fatigueTotal, fatigueHR, fatigueCad, fatigueGCT, fatigueImp | Fatigue breakdown |
| Coaching | aiResponse | Full Claude response text |

## 6.2 Export

- ⬇ Coach Log CSV — available on Done screen and History screen

- ⬇ This Run CSV — current run summary

- ⬇ All Runs CSV — complete run history

# 7. Key Algorithms

## 7.1 Fatigue Index (0–10)

HR component:      max(0, (HR - 143) / 20) × 3.5

Cadence component: max(0, 172 - cadence) × 0.6   [after 30s elapsed]

GCT component:     max(0, GCT - 245) × 0.05

Impact component:  max(0, impact - 2.1) × 4

Composite = HR×0.35 + Cadence×0.25 + GCT×0.20 + Impact×0.20

Note: Cadence fatigue is zero for the first 30 seconds to prevent phantom fatigue from a stationary foot pod reading.

## 7.2 GPS Distance Accumulation

Distance accumulates every second using the last known GPS pace — it does not freeze between GPS callbacks.

Every tick (1 second):

S.dist += 1 / gpsPace                    // accumulate each second

if gpsDist > S.dist: S.dist = gpsDist    // sync when GPS jumps ahead

Never goes backwards

First GPS fix: if gpsDist=0 and S.dist>0: gpsDist = S.dist  // no reset

## 7.3 Cadence (Foot Pod)

The ESP32 foot pod is strapped to one ankle, measuring single-foot cadence. The app doubles the received value for total (both feet) cadence.

BLE handler: if(cad > 0 && cad < 200) fpCadence = cad × 2

Result: 87spm raw → 174spm displayed ≈ Garmin 170-171spm ✓

## 7.4 Step Offset Correction

The ESP32 counts steps cumulatively since power-on. The app subtracts the offset recorded at run start to show per-run step count.

At run start: fpStepsOffset = fpSteps (current ESP32 count)

Each tick:    S.steps = max(0, fpSteps - fpStepsOffset)

If foot pod connects after run start: offset set on first BLE packet

## 7.5 HR Zones

Max HR: 185bpm

Z1: < 60% max  (< 111bpm)  — Recovery

Z2: 60-70%     (111-130)   — Aerobic

Z3: 70-80%     (130-148)   — Tempo

Z4: 80-90%     (148-167)   — Threshold

Z5: > 90%      (> 167bpm)  — Max

## 7.6 GPS Pace Staleness Detection

gpsPaceTime recorded whenever gpsPace updates

In tick(): gpsPaceStale = gpsPace>0 && (now - gpsPaceTime) > 15000

If stale: fall back to simPace(), show ⚡ SIM PACE, alert in GPS status bar

# 8. Runner Profile

| Attribute | Value |
| --- | --- |
| Name | Girish |
| Location | Mumbai, India |
| Total Runs | 323 on Garmin |
| Total Distance | 3,043 km |
| Longest Run | Full Marathon (Jan 2026) |
| Max HR | 185 bpm |
| Resting HR | 65 bpm |
| Baseline Cadence | 172 spm |
| Baseline GCT | 245 ms |
| Baseline Impact | 2.1 G |
| Typical Run Conditions | Warm & humid (Mumbai mornings) |
| Watch | Garmin Forerunner 245 |

# 9. Known Issues & Limitations

## 9.1 Screen Lock Kills App Timer

Android Doze mode suspends Chrome's setInterval when the screen locks. This stops the timer, GPS, and AI coaching. The Wake Lock API prevents auto-dimming but cannot override a manual power button press.

- Workaround: Set display timeout to 30 minutes (OnePlus 11R max). Use Run Shield to prevent accidental taps.

- Permanent fix: React Native app with Android ForegroundService (planned).

## 9.2 System Gestures Bypass Run Shield

Android system gestures (swipe-down notification panel, edge gestures, volume buttons) bypass Chrome entirely. The Run Shield cannot intercept these.

- Permanent fix: Native Android app in kiosk/lock-task mode (planned).

## 9.3 GCT Always 410ms (93% of Readings)

The current threshold-based firmware cannot detect a clean foot-lift signal at ankle position. The 400ms safety timeout fires on nearly every footstrike, producing a meaningless 410ms GCT reading.

- Confirmed by benchmark test: still/walk/jog impact data shows noise floor overlap.

- Fix: New firmware v2.0 planned with peak detection, complementary filter, gyroscope TC detection.

## 9.4 GPS Pace ±45s/km vs Garmin

Mumbai urban GPS conditions cause the app to read pace approximately 45 seconds per km faster than Garmin. Acceptable for coaching purposes — cadence and HR are accurate.

## 9.5 Distance Only ~51% Captured

Garmin recorded 14.1km on April 18. App captured 7.27km. Screen locked mid-run after 54 minutes.

# 10. Foot Pod Firmware

## 10.1 Current Firmware v1.1

| Parameter | Value |
| --- | --- |
| Sampling Rate | 100Hz (every 10ms) |
| Impact Threshold (start) | 1.2G |
| Exit Threshold | 0.9G (1.2 × 0.75) |
| Min Step Interval | 200ms |
| Max Strike Duration | 400ms (safety timeout) |
| Smoothing Window | 5 samples (50ms) |
| Cadence Buffer | Last 6 step intervals |
| BLE Broadcast | Every 1 second |
| Accelerometer Range | ±2G default (MPU6050_light library) |

## 10.2 Known Firmware Issues

- GCT always 410ms — safety timeout fires on every strike

- Steps counting while stationary — threshold too low for ankle noise floor

- Accelerometer range ±2G may clip impact peaks (jogging shows 3.08G peaks)

## 10.3 Planned Firmware v2.0

Based on benchmark test data (still/walk/jog impact analysis) and research into running biomechanics:

- ±8G accelerometer range — prevents signal clipping during running

- ±2000°/s gyroscope range — enables Terminal Contact detection

- DLPF setting 3 (44Hz) — removes vibration noise, keeps impact spikes

- X-gyro clock source — more stable than internal oscillator

- Static calibration (1000 samples) — removes axis offsets, dynamic threshold

- Complementary filter — stable foot orientation from accel + gyro fusion

- Peak detection on squared acceleration signal — better noise separation

- Gyroscope-based Terminal Contact detection — accurate GCT measurement

- Calibration protocol: 10 seconds still after power-on, threshold auto-set

## 10.4 Benchmark Test Results

Testing conducted April 19, 2026 with pod strapped to ankle under sock:

| Phase | Avg Impact | Max Impact | Conclusion |
| --- | --- | --- | --- |
| Completely still | 1.48G | 2.17G | Noise floor baseline |
| Walking | 1.42G | 2.52G | Overlaps with still — unreliable |
| In-place jog | 1.53G | 2.22G | Same as still — not representative |
| Forward running | 2.61G | 3.08G | 1.76× above still — detectable |

Conclusion: Walking cannot be reliably detected at ankle position. Forward running shows sufficient separation (1.76×). New firmware threshold: 2.4G minimum peak.

# 11. Version History

| Version | Changes |
| --- | --- |
| v7.0 | Ground-up architecture rewrite — fixed position:fixed click-blocking |
| v7.1 | Wake lock, GPS label accuracy fix |
| v7.2 | Coaching log feature (cross-run aggregation, 23 fields) |
| v7.3 | GPS distance/pace calculation bugs fixed |
| v7.4 | Foot pod BLE integration, step offset correction |
| v8.0 | GPS pre-warm (getCurrentPosition), all metrics sim indicators |
| v8.1 | paceSim used before declaration bug fix, fragile querySelector replaced |
| v8.2 | GPS staleness detection (15s), accuracy 100m→150m, movement 5m→3m |
| v9.0 | Run Shield feature — hold 2s to unlock |
| v9.1 | Lock button (🔒) added to live screen header |
| v9.2 | API key moved to localStorage, cadence ×2 fix for single-foot pod |
| v9.3 | Current stable version |

# 12. Future Roadmap

## 12.1 Immediate (Firmware v2.0)

- Peak-detection firmware with calibration, complementary filter, gyroscope TC

- Accurate GCT measurement replacing current 410ms timeout

## 12.2 Near Term

- Battery voltage indicator (requires 2× resistor voltage divider on GPIO34)

- Battery time estimator (time-based until resistors arrive)

- LiPo 1000mAh upgrade — doubles runtime to ~6 hours

- ESP32 light sleep mode — extends standby to 12+ hours

## 12.3 React Native App (Major)

- Solves screen lock problem — Android ForegroundService keeps timer alive

- Solves system gesture problem — kiosk/lock-task mode available

- Full background GPS and BLE — no browser limitations

- Same BLE UUIDs and data format — foot pod firmware unchanged

- Same Claude AI coaching logic — ported from JS

- Coach log and run history — React Native AsyncStorage

## 12.4 Post-Run AI Analysis

- Feed entire coach log to Claude after run

- Pattern recognition across multiple runs

- Personalised training recommendations based on longitudinal data

- Fatigue signature identification — at what exact combination does form break down?